MODULAR NITROGEN GENERATOR

Introduction

EPSEA Modular Nitrogen Generator uses multiple high-strength aluminum alloy cavities filled with high-performance molecular sieve to separate nitrogen and oxygen from compressed air by pressure swing adsorption (PSA). It delivers nitrogen of 95~99.999% purity at a flow of 2~280Nm³/h, with a low air-to-nitrogen ratio of 4.2:1 at 99.99% purity — about 15% more energy-efficient than traditional twin-tower generators. PLC programmable control with HMI real-time monitoring enables automatic evacuation of unqualified nitrogen and energy-saving automatic pause, while the non-corrosive aluminum alloy structure ensures a long service life.

| Nitrogen Flow | 2-280 Nm³/h |
| --- | --- |
| Inlet Temp. | ≤ 35 ℃ |
| Inlet Pressure | 0.6-1.0 MPa |
| Nitrogen Purity | 95-99.999% |
| Noise | ≤ 80 dB |
| PDP | -40 ℃ ~ -70 ℃ |
| Power Supply | AC220V / AC110V DC24V 50/60Hz |
| Nitrogen Outlet Pressure | 0.1-0.8 MPa (Adjustable, with built-in nitrogen tank) |
| Max Ambient Temp. | ≤ 45 ℃ |
| Control Method | PLC programmable control / HMI microcomputer control |

Features of Modular Nitrogen Generator

◆  High efficiency & low air consumption: at 99.99% purity the air-to-nitrogen ratio is 4.2:1 versus 5:1 for traditional twin-tower units, saving about 15% energy consumption.

◆  PLC control with HMI: monitors air/nitrogen pressure, purity and flow in real time; automatically purges off-spec nitrogen and supports energy-saving pause.

◆  VP control logic (standard): adapts to load fluctuations and adjusts switching cycles, reducing air consumption by 15-50% versus rated values under specific conditions.

◆  High-strength aluminum alloy structure: corrosion- and rust-proof, avoiding failure from internal pipe corrosion; some models with a built-in nitrogen tank save installation space.

◆  24 h continuous supply & easy expansion: runs continuously to avoid downtime from cylinder changeover; when capacity grows, simply add modules without replacing the whole machine.

Note: Other special standards can be customized. For more details, please contact us.