TWIN TOWER NITROGEN GENERATOR

Introduction

The Twin Tower Nitrogen Generator adopts an advanced energy-saving inequality equalization structure, improving adsorbent utilization and directly reducing compressed air consumption. Its optimized internal structure distributes air flow evenly, reducing impact on the adsorbent and extending service life.

Working with well-known carbon molecular sieve manufacturers, the most efficient ratio can be selected according to actual conditions, while reliable air source treatment accessories ensure stable operation. The integrated filling process — combining vacuum automatic feeding, blizzard filling and vibration — achieves denser packing, increasing filling weight by over 10% for the same volume.

The unit features domestic stainless steel pneumatic angle seat valves with fast response, low energy consumption, good sealing and long service life, plus a spring compression device that automatically compensates for molecular sieve reduction to prevent powdering. Impure nitrogen automatic / manual evacuation (optional) and components from famous brands ensure stable operation and nitrogen quality.

| Nitrogen Purity | 95-99.999% |
| --- | --- |
| Max. Intake Air Temp. | 38 ℃ |
| Min. Intake Air Temp. | 1 ℃ |
| Inlet Pressure | 0.6-0.8 MPa |
| Max. Ambient Temp. | ≤ 45 ℃ |
| Power Requirements | 220V 50/60Hz / 110V DC24V |
| Noise | ≤ 80 dB |
| Nitrogen Dew Point | ≤ -40 ℃ |
| Control Type (Standard) | PLC programmable control |
| Control Type (High Match) | Human-machine touch interface |

Features

◆  Energy-saving equalization structure: advanced structure improves adsorbent utilization and directly reduces compressed air consumption.

◆  Optimized internal structure: ensures evenly distributed air flow, reduces impact on the adsorbent and extends its service life.

◆  Quality molecular sieve: cooperates with well-known carbon molecular sieve manufacturers at home and abroad to select the most energy-efficient ratio according to actual working conditions.

◆  Reliable air-source treatment: quality accessories ensure stable operation and long service life of the equipment.

◆  Integrated filling process: vacuum automatic feeding, blizzard filling and vibration platform together ensure uniform and dense packing; for the same volume the filling weight increases by over 10%, enhancing long-term reliability.

◆  Stainless steel pneumatic angle seat valve: fast opening/closing, low energy consumption, good sealing, easy installation and maintenance, long service life.

◆  Spring compression device: automatically compensates when the molecular sieve settles, eliminating powdering and prolonging service life.

◆  Impure nitrogen evacuation: automatic / manual evacuation device (optional).

◆  Brand-name components: components from famous domestic and international brands ensure stable operation and nitrogen quality.