MODULAR OXYGEN GENERATOR

EPSEA Modular Oxygen Generator uses multiple high-strength aluminum alloy cavities filled with adsorbent to separate nitrogen and oxygen from compressed air by pressure swing adsorption (PSA). It delivers oxygen at (93±3)% purity with a flow of 1–27Nm³/h and a dew point of -40~-70°C, with low air-oxygen ratio and low production cost.

Technical Indicators

| Item | Specification |
| --- | --- |
| Oxygen Flow Rate Range | 1-27Nm³/H |
| Oxygen Purity Range | (93±3) % |
| Oxygen Pressure Range | 0.1-0.5MPa |
| Oxygen Dew Point Range | -40~-70°C |
| Noise | ≤80dB |
| Power Supply | AC220V/50/60HZ-0.3KW |
| Inlet Temperature Range | 15-35°C |
| Ambient Temperature Range | 0-45°C |
| Control Mode | PLC Automatic Operation |

Product Features

High efficiency, low air oxygen ratio — Reduces oxygen production costs, with the production cost of 1 Nm³ oxygen being on more than 1.3 KW*H.

PLC and HMI control — Real-time detection and recording of operating data. Trends for purity, flow rate and pressure can be recorded for over 30 days. High level of automation, enabling one-button start/stop and simple operation.

Corrosion-resistant aluminum alloy structure — Non-corrosive and rust-proof, avoiding equipment failure caused by corrosion and shedding of the inner pipeline, ensuring normal production and greatly extending the service life of the whole machine. Some models can be built-in with an oxygen tank, saving the most effective installation space.

VP Control Logic (standard) — EPSEA self-developed VP Control Logic adaptively responds to load changes and automatically adjusts the switching cycle. Under specific operating conditions, it reduces air consumption by 15–50% compared with the rated flow load, cutting energy consumption and achieving energy savings and emission reductions.

Not classified as a pressure vessel — Saves time and costs on subsequent special equipment qualification review and application.