TWIN TOWER OXYGEN GENERATOR

EPSEA Twin Tower Oxygen Generator uses a twin-tower PSA structure with high-performance carbon molecular sieve to separate nitrogen and oxygen from compressed air. It delivers oxygen at (93±3)% purity with a flow of 10–300Nm³/h and a dew point of -40~-70°C, with a reasonable internal structure and reliable long-term operation.

Technical Indicators

| Item | Specification |
| --- | --- |
| Oxygen Flow Rate Range | 10-300Nm³/H |
| Oxygen Purity Range | (93±3) % |
| Air Inlet Pressure | 0.4-0.6MPa |
| Oxygen Pressure Range | 0.1-0.5MPa |
| Oxygen Dew Point Range | -40~-70°C |
| Noise | ≤80dB |
| Power Supply | AC220V/50/60HZ-0.3KW |
| Inlet Temperature Range | 15-35°C |
| Ambient Temperature Range | 0-45°C |
| Control Mode | PLC Automatic Operation |

Product Features

Even airflow distribution — The advanced internal structure ensures that the air flow is evenly distributed, reduces the impact on the adsorbent, and increases the service life of the adsorbent.

Optimized molecular sieve ratio — In cooperation with well-known carbon molecular sieve manufacturers, the most energy-efficient and efficient ratio can be selected according to actual working conditions.

Reliable air source treatment — Reliable air source treatment accessories ensure the stable operation and service life of the equipment. Continuous 24-hour operation for no less than 5 years.

Integrated filling process — Combining vacuum automatic feeding, blizzard filling and vibration platform ensures more uniform and dense adsorption material packing. This minimizes the friction coefficient, increases the filling weight by over 10% for the same volume, and enhances the long-term operational reliability of the entire unit/molecular sieve.

Siemens PLC control — It adopts Siemens PLC programmable controller for automatic control; one-button start does not require special personnel to operate, and can reserve connections with remote devices such as computers and mobile phones, and observe the running status in real time.

Special spring compression device — Used when the volume of the molecular sieve in the adsorption tower decreases, it automatically compensates to prevent the molecular sieve from pulverizing and extend its service life.

Impure oxygen evacuation — Use device of impure oxygen automatic evacuation / manual evacuation device.