HEATLESS MODULAR DESICCANT AIR DRYER

Introduction

EPSEA Heatless Modular Desiccant Air Dryer uses multiple high-strength aluminum cavities. Each cavity is divided into two chambers, each filled with adsorbent, so that the wet compressed air is dried while the corresponding chamber is regenerated by variable-pressure adsorption. When regenerating, part of the dry compressed air expands from working pressure to atmospheric pressure, passes through the water-saturation adsorbent, takes away the moisture, and makes the adsorbent regenerate.

The modular construction uses aluminum alloy and stainless steel bolts, avoiding secondary pollution from carbon steel rust in contact with compressed air. High utilization of the machine design ensures a constant and stable pressure dew point, making compressed air cleaner and solving gas quality problems in production.

Features

Low dew point — Aluminum alloy and stainless steel bolts avoid secondary rust pollution; high desiccant utilization ensures a constant and stable pressure dew point.

Low loss consumption — Modular structure with separate diffusion per module; high-quality adsorbent plus intelligent control minimizes regeneration gas consumption.

Long service life — Global reputed pneumatic components, scientific sealing design, and anti-oxidation / anti-corrosion treatment contribute to 30-year service life.

Technical Indicators

Standard condition

| Item | Specification |
| --- | --- |
| Inlet air temperature | ≤38°C |
| Ambient temperature | ≤38°C |
| Working pressure | 0.7MPa |

The scope of work

| Item | Specification |
| --- | --- |
| Max inlet air temperature | 50°C |
| Ambient temperature | 45°C |
| Working pressure | 0.4–1.0MPa |

Technical indicators

| Item | Specification |
| --- | --- |
| Pressure dew point | -20°C ~ -40°C |
| Pressure loss | ≤0.021MPa |